Wound measurement – working out surface area in cm²

One of the most effective ways of predicting whether a wound is likely to be healing in a ‘normal’ way is by measuring wound size reduction over a period of time. In normal wound healing there should have been a minimum of a 40% reduction in wound size after 6 weeks.

To determine this, it is essential that a baseline measurement is taken at the initial wound assessment and followed up at regular reassessments.

Due to the irregular shape of wounds it is not possible to accurately measure the changes in wound size by taking a length and width calculation. In routine clinical practice there are 2 main ways of obtaining a more accurate measurement. 1) Wound circumference 2) Surface area in cm². To get an exact measurement you need computerised imagery but unfortunately we haven’t got that!!

As measuring circumference is ‘practically’ rather difficult (unless you have a piece of string!) we recommend that surface area in cm² be used.

Working out surface area in cm²

- Using an opsite flexigrid, trace around the edges of the wound using a fine tipped permanent marker pen. (OPsite flexigrids can be obtained through e procurement and come in a range of sizes 6cmx7cm, 12cmx25cm, 10cmx12cm.  Please use the size most appropriate to the wound size. Pens can also be purchased this way).

- The contact layer is slightly adherent (like a film dressing) which is removed after the tracing is taken avoiding the need to wipe away exudates etc before the tracing is put in the patients notes. (If the patient has very fragile, painful skin, you may wish to remove this layer before tracing the wound)

- With very large wounds you may need to use more than one piece of flexigrid, this can be  a little ‘challenging’ but can be done (See figure 2)

- Write patients name/ initials, the date tracing was taken and where anatomically the tracing has come from (i.e. (L) leg ulcer – medial side)

Figure 1. Marking out the cm²

- Place the tracing over a plain light coloured surface (i.e. piece of paper).

- Firstly, using the marker pen highlight all the cm² within the margin of the wound - these compose of 4 small squares (see figure 1).

- As you see, you have mainly whole squares, but around the edges there will be a number of incomplete squares.

- Firstly, count the number of whole squares – make a note of this (in figure 1 this is 14), then…

- Convert the incomplete squares into whole ones (i.e. there may be 4 small pieces that together would make a complete cm²). This possibly won’t add up to a whole number but that’s fine, in figure 1 this is 7.75cm². Add this number to your number of whole squares to give a total surface area figure in cm².

So, taking the tracing above, there were 14 whole squares and 7.75 squares made up from the incomplete bits which together totalled 21.75. Therefore 21.75cm² would be your baseline surface area. Document this on your wound assessment form

Determining the percentage reduction in wound surface area

- Re trace the wound as previously explained.

- Work out the surface area in cm². Say for example the new surface area is 14.cm²

- Work out the reduction in surface area by using the following equation:

New surface area (14) ÷ last surface area (21.75) x 100 = 64.36%

4. Next take the % figure (64.36) from 100 (100 – 64.36 = 35.6%)

This means that there has been a 35.6% reduction in wound area since the last measurement

Figure 2. Using multiple flexigrids to measure large wounds.